KD-5230 User manual V2.6

Contents

Factory Defaults	9

Custom Defaults	10

Basic Settings	11

Interface	11

RS232 Baud Rate	12

RS232 Data Bits	13

RS232 Parity	14

RS232 Stop Bits	15

Keyboard Layout	16

Scan Mode	21

Sense Mode Sensitivity	22

Decode Redundancy	23

Decode Area	25

Decode Session Timeout	28

Time to Read Same Barcode	29

Time to Suspend State	31

Illumination Mode	32

Illumination Level	33

Aiming Pattern	34

Good Read Beep	35

Good Read Beep Frequency	36

Symbologies	37

Enable/Disable All Symbologies	37

Enable 1D/2D Symbologies	38

UPC-A	39

Enable/Disable UPC-A	39

Number System Transmission	40

Check Digit Transmission	41

Expand UPC-A to EAN13	42

UPC-A 2/5-Digit Add-ons	43

UPC-E	46

Enable/Disable UPC-E	46

Number System Transmission	47

Check Digit Transmission	48

Expand UPC-E to UPC-A	49

UPC-E 2/5-Digit Add-ons	50

EAN 13	53

Enable/Disable EAN 13	53

Check Digit Transmission	54

ISBN	55

EAN 13 2/5-Digit Add-ons	56

EAN 8	59

Enable/Disable EAN 8	59

Check Digit Transmission	60

Expand EAN 8 to EAN 13	61

EAN 8 2/5-Digit Add-ons	62

Code 128 / GS1-128	65

Enable/Disable Code 128 / GS1-128	65

GS1-128 AIM ID	66

Set Lengths for Code 128	67

Code 39	69

Enable/Disable Code 39	69

Code 39 Full ASCII	70

Check Character Calculation	71

Check Character Transmission	72

Start / Stop Characters	73

Set Lengths for Code 39	74

Code 93	76

Enable/Disable Code 93	76

Set Lengths for Code 39	77

Code 32	79

Enable/Disable Code 32	79

Pharmacode	80

Enable/Disable Pharmacode	80

Codabar	81

Enable/Disable Codabar	81

Check Character Verification	82

Check Character Transmission	83

Start / Stop Characters	84

Set Lengths for Codabar	85

MSI	87

Enable/Disable MSI	87

Check Character Calculation	88

Check Character Transmission	89

Check Character Algorithm	90

Set Lengths for MSI	91

Interleaved 2 of 5	93

Enable/Disable Interleaved 2 of 5	93

Check Character Calculation	94

Check Character Transmission	95

Set Lengths for Interleaved 2 of 5	96

GS1 DataBar 14	98

Enable/Disable GS1 DataBar 14	98

Application Identifier Transmission	99

GS1 DataBar 14 Stacked	100

Eeable/Disable GS1 DataBar 14 Stacked	100

Application Identifier Transmission	101

GS1 DataBar Expanded	102

Enable/Disable GS1 DataBar Expanded	102

Application Identifier Transmission	103

GS1 DataBar Expanded Stacked	104

Enable/Disable GS1 DataBar Expanded Stacked	104

Application Identifier Transmission	105

GS1 DataBar Limited	106

Enable/Dsiable GS1 DataBar Limited	106

Application Identifier Transmission	107

GS1 Composite Component A	108

Enable/Disable GS1 Composite Component A	108

GS1 Composite Component B	109

Enable/Disable GS1 Composite Component B	109

GS1 Composite Component C	110

Enable/Disable GS1 Composite Component C	110

PDF417	111

Enable/Disable PDF417	111

Micro PDF417	112

Enable/Disable Micro PDF417	112

Data Matrix	113

Enable/Disable Data Matrix	113

GS1 Data Matrix AIM ID	114

QR	115

Enable/Disable QR	115

GS1 QR AIM ID	116

Unicode Keyboard Output	117

Micro QR	127

Enable/Disable Micro QR	127

Aztec	128

Enable/Disable Aztec	128

MaxiCode	129

Enable/Disable MaxiCode	129

DotCode	130

Enable/Disable DotCode	130

GS1 DotCode AIM ID	131

Data Editing	132

Data Format	132

Prefix/Suffix	132

Truncate Data	135

Set Data for Codes	137

AIM ID	144

Keyboard Function Key Mapping	145

Keyboard Caps Lock State	147

Case Conversion	148

Control Characters Conversion	149

Appendix A - ASCII Codes	166

Appendix B - Direct Keys	198

Appendix C - Digit Number	211

Revision History	212

Factory Defaults

Start / End

Scanning the following barcode can restore the scanner to the factory defaults

Factory Defaults

Custom Defaults

Start / End

Scanning the Save as Custom Defaults barcode can set the current settings as custom defaults. Scanning the Restore Custom Defaults barcode can reset all parameters to the custom defaults.

Save as Custom Defaults

Restore Custom Defaults

Basic Settings

| Start / End |  |
| --- | --- |
| Interface |  |
| USB Keyboard / DEFAULT | RS232 COM Port |
| USB Virtual COM Port |  |

| Start / End |  |
| --- | --- |
| RS232 Baud Rate |  |
| 9600 / DEFAULT | 19200 |
| 38400 | 57600 |
| 115200 | 230400 |

Start / End

RS232 Data Bits

7 data bits

8 data bits / DEFAULT

| Start / End |  |
| --- | --- |
| RS232 Parity |  |
| No Parity | Odd Parity |
| Even Parity |  |

Start / End

RS232 Stop Bits

1 Stop bit / DEFAULT

2 Stop bit

| Start / End |  |
| --- | --- |
| Keyboard Layout Keyboard layouts vary from country to country. The default setting is U.S. keyboard. |  |
| U.S. / DEFAULT | Belgium |
| Britain | Brazil |

| Start / End |  |
| --- | --- |
| Keyboard Layout - Continued |  |
| Czech Republic | Denmark |
| France | Germany |

| Start / End |  |
| --- | --- |
| Keyboard Layout - Continued |  |
| Hungary | Italy |
| Japan | Norway |

| Start / End |  |
| --- | --- |
| Keyboard Layout - Continued |  |
| Poland | Portugal |
| Romania | Spain |

| Start / End |  |
| --- | --- |
| Keyboard Layout - Continued |  |
| Sweden | Slovakia |
| Turkish Q |  |

| Start / End |  |
| --- | --- |
| Scan Mode |  |
| Trigger Mode / DEFAULT | Sense Mode |
| Continuous Mode |  |

| Start / End |  |
| --- | --- |
| Sense Mode Sensitivity |  |
| Low Sensitivity | Medium Sensitivity / DEFAULT |
| High Sensitivity |  |

.

Decode Redundancy

The scanner offers three levels of decode redundancy. Select higher redundancy levels for decreasing levels of bar code quality.

As redundancy levels increase, the scanner's aggres-siveness decreases. Select the redundancy level appropriate for the bar code quality.

Redundancy Level 1

All code types just read one time.

Redundancy Level 2

All code types must be successfully read two times before being decoded.

Redundancy Level 3

All code types must be successfully read three times before being decoded.

| Start / End |  |
| --- | --- |
| Decode Redundancy - Continued |  |
| 1 time / DEFAULT | 2 times |
| 3 times |  |

Decode Area

The scanner offers four settings of decode area.

Full size of image

To decode the barcode within full size of image

Decode Area (100% of image)

75% of image

To decode the barcode within 75% of image

Ignore Area (12.5% of image)

Decode Area (75% of image)

Ignore Area (12.5% of image)

Decode Area - Continued

50% of image

To decode the barcode within 50% of image

Ignore Area (25% of image)

Decode Area (50% of image)

Ignore Area (25% of image)

25% of image

To decode the barcode within 25% of image

Ignore Area (37.5% of image)

Decode Area (25% of image)

Ignore Area (37.5% of image)

| Start / End |  |
| --- | --- |
| Decode Area - Continued |  |
| Full Size / DEFAULT | 75% of image |
| 50% of image | 25% of image |

Decode Session Timeout

This parameter sets the maximum time decode session continues during a scan attempt. This feature is only applicable to the Trigger and Sense modes. It is programmable in 1ms increments from 1ms to 60,000 ms. When it is set to 0 , the timeout is infinite. The default setting is 5,000 ms.

Set the decode session timeout to 1,500 ms

1. Scan the Start barcode.

2. Scan the Decode Session Timeout barcode.

3. Scan the “1”, “5”, “0” and “0” barcodes from the Digit Number in Appendix C.

4. Scan the End barcode.

Start / End

Decode Session Timeout

Time to Read Same Barcode

- Timeout between Decodes

Timeout between Decodes (Same Barcode) can avoid undesired rereading of same barcode in a given period of time. This feature is only applicable to the Sense and Continuous modes.

It is programmable in 1ms increments from 1ms to 5,000 ms. When it is set to 0, the timeout is disable. The default setting is 1500 ms.

- Ignore Same Code

Time to ignore the decode when read same barcode in a given period of time. This feature is only applicable to the Sense and Continuous modes.

Set the timeout between decodes to 500ms

1. Scan the Start barcode.

2. Scan the Timeout between Decodes barcode

3. Scan the “5”, “0” and “0” barcodes from the Digit Number in Appendix C.

4. Scan the End barcode.

Set the ignore same barcode to 250 ms

1. Scan the Start barcode.

2. Scan the Ignore Same Barcode

3. Scan the “2”, “5” and “0” barcodes from the Digit Number in Appendix C.

4. Scan the End barcode.

Start / End

Time to Read Same Barcode - Continued

Timeout between Decodes / DEFAULT

Ignore Same Barcode

Time to Suspend State

This parameter sets the time to enter to suspend state when the decoder is idle. This feature is only applicable to the Trigger and Continuous modes. It is programmable in 1 ms increments from 1ms to 36,00,000 ms. When it is set to 0, the timeout is disable. The default setting is 15,000 ms.

Set the time to suspend state to 2,500 ms

1. Scan the Start barcode.

2. Scan the Time to Suspend State barcode

3. Scan the “2”, “5”, “0” and “0” barcodes from the Digit Number in Appendix C.

4. Scan the End barcode.

Start / End

Time to Suspend State

| Start / End |  |
| --- | --- |
| Illumination Mode |  |
| Disable | Enable by trigger / DEFAULT |
| Always On | Fade Up |

| Start / End |  |
| --- | --- |
| Illumination Level |  |
| Minimum | Medium |
| Maximum / DEFAULT |  |

| Start / End |  |
| --- | --- |
| Aiming Pattern |  |
| Disable | Enable by trigger / DEFAULT |
| Always On |  |

Start / End

Good Read Beep

Disable

Enable / DEFAULT

| Start / End |  |
| --- | --- |
| Good Read Beep Frequency |  |
| Low | Medium / DEFAULT |
| High |  |

Symbologies

Start / End

Enable/Disable All Symbologies

Enable All Symbologies

Disable All Symbologies

Start / End

Enable 1D/2D Symbologies

Enable 1D Symbologies

Enable 2D Symbologies

UPC-A

Start / End

Enable/Disable UPC-A

Enable UPC-A / DEFAULT

Disable UPC-A

Start / End

Number System Transmission

Enable Number System Transmission / DEFAULT

Disable Number System Transmission

Start / End

Check Digit Transmission

Send Check Digit / DEFAULT

Don't Send Check Digit

Start / End

Expand UPC-A to EAN13

Don't Expand to EAN13 / DEFAULT

Expand to EAN13

Start / End

UPC-A 2/5-Digit Add-ons

Disable UPC-A Add-ons / DEFAULT

Enable UPC-A 2/5-Digit Add-ons

Start / End

UPC-A 2/5-Digit Add-ons - Continued

Enable UPC-A 2-Digit Add-ons

Enable UPC-A 5-Digit Add-ons

Start / End

UPC-A 2/5-Digit Add-ons - Continued When UPC-A Add-ons Only is selected, the scanner will only read UPC-A barcodes that contain add-on codes.

Disable UPC-A Add-ons Only / DEFAULT

Enable UPC-A Add-ons Only

UPC-E

Start / End

Enable/Disable UPC-E

Enable UPC-E / DEFAULT

Disable UPC-E

Start / End

Number System Transmission

Enable Number System Transmission / DEFAULT

Disable Number System Transmission

Start / End

Check Digit Transmission

Send Check Digit / DEFAULT

Don't Send Check Digit

Start / End

Expand UPC-E to UPC-A

Don't Expand to UPC-A / DEFAULT

Expand to UPC-A

Start / End

UPC-E 2/5-Digit Add-ons

Disable UPC-E Add-ons / DEFAULT

Enable UPC-E 2/5-Digit Add-ons

Start / End

UPC-E 2/5-Digit Add-ons - Continued

Enable UPC-E 2-Digit Add-ons

Enable UPC-E 5-Digit Add-ons

Start / End

UPC-E 2/5-Digit Add-ons - Continued When UPC-E Add-ons Only is selected, the scanner will only read UPC-A barcodes that contain add-on codes.

Disable UPC-E Add-ons Only / DEFAULT

Enable UPC-E Add-ons Only

EAN 13

Start / End

Enable/Disable EAN 13

Enable EAN 13 / DEFAULT

Disable EAN 13

Start / End

Check Digit Transmission

Send Check Digit / DEFAULT

Don't Send Check Digit

Start / End

ISBN

Disable ISBN / DEFAULT

Enable ISBN

Start / End

EAN 13 2/5-Digit Add-ons

Disable EAN 13 Add-ons / DEFAULT

Enable EAN 13 2/5-Digit Add-ons

Start / End

EAN 13 2/5-Digit Add-ons - Continued

Enable EAN 13 2-Digit Add-ons

Enable EAN 13 5-Digit Add-ons

Start / End

EAN 13 2/5-Digit Add-ons - Continued When EAN 13 Add-ons Only is selected, the scanner will only read UPC-A barcodes that contain add-on codes.

Disable EAN 13 Add-ons Only / DEFAULT

Enable EAN 13 Add-ons Only

EAN 8

Start / End

Enable/Disable EAN 8

Enable EAN 8 / DEFAULT

Disable EAN 8

Start / End

Check Digit Transmission

Send Check Digit / DEFAULT

Don't Send Check Digit

Start / End

Expand EAN 8 to EAN 13

Don't Expand to EAN 13 / DEFAULT

Expand to EAN 13

Start / End

EAN 8 2/5-Digit Add-ons

Disable EAN 8 Add-ons / DEFAULT

Enable EAN 8 2/5-Digit Add-ons

Start / End

EAN 8 2/5-Digit Add-ons - Continued

Enable EAN 8 2-Digit Add-ons

Enable EAN 8 5-Digit Add-ons

Start / End

EAN 8 2/5-Digit Add-ons - Continued When EAN 8 Add-ons Only is selected, the scanner will only read UPC-A barcodes that contain add-on codes.

Disable EAN 8 Add-ons Only / DEFAULT

Enable EAN 8 Add-ons Only

Code 128 / GS1-128

Start / End

Enable/Disable Code 128 / GS1-128

Enable Code 128 / GS1-128 / DEFAULT

Disable Code 128 / GS1-128

Start / End

GS1-128 AIM ID

Transmit GS1-128 AIM ID

Do Not Transmit GS1-128 AIM ID / DEFAULT

Set Lengths for Code 128 - One Discrete Length Select this option to decode the symbol containing a selected length. Select the length using the numeric bar codes in ASCII Code. For example, to decode only Code 128 symbols with 14 characters, scan Code 128 One Discrete Length, then scan 1 followed by 4. - Two Discrete Lengths Select this option to decode the symbol containing either of two selected lengths. Select lengths using the numeric bar codes in ASCII Code. For example, to decode only Code 128 symbols containing either 2 or 14 characters, select Code 128 Two Discrete Lengths, then scan 0, 2, 1, and then 4. - Length Within Range Select this option to decode the symbol with a specific length range. Select lengths using numeric bar codes in ASCII Code. For example, to decode Code 128 symbols containing between 4 and 12 characters, first scan Code 128 Length Within Range. Then scan 0, 4, 1, and 2. - Any Length Select this option to decode the symbol containing any number of characters within the digital scanner’s capability.

| Start / End |  |
| --- | --- |
| Set Lengths for Code 128 - Continued |  |
| One Discrete Length | Two Discrete Lengths |
| Length Within Range | Any Length / DEFAULT |

Code 39

Start / End

Enable/Disable Code 39

Enable Code 39 / DEFAULT

Disable Code 39

Start / End

Code 39 Full ASCII

Disable Code 39 Full ASCII / DEFAULT

Enable Code 39 Full ASCII

Start / End

Check Character Calculation

Disable Check Char Calculation / DEFAULT

Enable Check Char Calculation

Start / End

Check Character Transmission

Disable Check Char Transmission / DEFAULT

Enable Check Char Transmission

Start / End

Start / Stop Characters

Don't Transmit Start / Stop Characters / DEFAULT

Transmit Start / Stop Characters

Set Lengths for Code 39

- One Discrete Length

Select this option to decode the symbol containing a selected length.

Select the length using the numeric bar codes in ASCII Code. For example, to decode only Code 39 symbols with 14 characters, scan Code 39 One Discrete Length, then scan 1 followed by 4.

- Two Discrete Lengths

Select this option to decode the symbol containing either of two selected lengths. Select lengths using the numeric bar codes in ASCII Code. For example, to decode only Code 39 symbols containing either 2 or 14 characters, select Code 39 Two Discrete Lengths, then scan 0, 2, 1, and then 4.

- Length Within Range

Select this option to decode the symbol with a specific length range.

Select lengths using numeric bar codes in ASCII Code. For example, to decode

Code 39 symbols containing between 4 and 12 characters, first scan Code 39 Length Within Range. Then scan 0, 4, 1, and 2.

- Any Length

Select this option to decode the symbol containing any number of characters within the digital scanner’s capability.

| Start / End |  |
| --- | --- |
| Set Lengths for Code 39 - Continued |  |
| One Discrete Length | Two Discrete Lengths |
| Length Within Range | Any Length / DEFAULT |

Code 93

Start / End

Enable/Disable Code 93

Enable Code 93 / DEFAULT

Disable Code 93

Set Lengths for Code 93

- One Discrete Length

Select this option to decode the symbol containing a selected length.

Select the length using the numeric bar codes in ASCII Code. For example, to decode only Code 93 symbols with 14 characters, scan Code 93 One Discrete Length, then scan 1 followed by 4.

- Two Discrete Lengths

Select this option to decode the symbol containing either of two selected lengths. Select lengths using the numeric bar codes in ASCII Code. For example, to decode only Code 93 symbols containing either 2 or 14 characters, select Code 93 Two Discrete Lengths, then scan 0, 2, 1, and then 4.

- Length Within Range

Select this option to decode the symbol with a specific length range.

Select lengths using numeric bar codes in ASCII Code. For example, to decode

Code 93 symbols containing between 4 and 12 characters, first scan Code 93 Length Within Range. Then scan 0, 4, 1, and 2.

- Any Length

Select this option to decode the symbol containing any number of characters within the digital scanner’s capability.

| Start / End |  |
| --- | --- |
| Set Lengths for Code 93 - Continued |  |
| One Discrete Length | Two Discrete Lengths |
| Length Within Range | Any Length / DEFAULT |

Code 32

Start / End

Enable/Disable Code 32

Enable Code 32

Disable Code 32 / DEFAULT

Pharmacode

Start / End

Enable/Disable Pharmacode

Enable Pharmacode

Disable Pharmacode / DEFAULT

Codabar

Start / End

Enable/Disable Codabar

Enable Codabar

Disable Codabar / DEFAULT

Start / End

Check Character Verification

Disable Check Char Verification / DEFAULT

Enable Check Char Verification

Start / End

Check Character Transmission

Disable Check Char Transmission / DEFAULT

Enable Check Char Transmission

Start / End

Start / Stop Characters

Transmit Start / Stop Characters

Don't Transmit Start / Stop Characters / DEFAULT

Set Lengths for Codabar

- One Discrete Length

Select this option to decode the symbol containing a selected length.

Select the length using the numeric bar codes in ASCII Code. For example, to decode only Codabar symbols with 14 characters, scan Codabar One Discrete Length, then scan 1 followed by 4.

- Two Discrete Lengths

Select this option to decode the symbol containing either of two selected lengths. Select lengths using the numeric bar codes in ASCII Code. For example, to decode only Codabar symbols containing either 2 or 14 characters, select Codabar Two Discrete Lengths, then scan 0, 2, 1, and then 4.

- Length Within Range

Select this option to decode the symbol with a specific length range.

Select lengths using numeric bar codes in ASCII Code. For example, to decode

Codabar symbols containing between 4 and 12 characters, first scan Codabar Length Within Range. Then scan 0, 4, 1, and 2.

- Any Length

Select this option to decode the symbol containing any number of characters within the digital scanner’s capability.

| Start / End |  |
| --- | --- |
| Set Lengths for Codabar - Continued |  |
| One Discrete Length | Two Discrete Lengths |
| Length Within Range | Any Length / DEFAULT |

MSI

Start / End

Enable/Disable MSI

Enable MSI

Disable MSI / DEFAULT

Start / End

Check Character Calculation

Enable Check Char Calculation / DEFAULT

Disable Check Char Calculation

Start / End

Check Character Transmission

Enable Check Char Transmission / DEFAULT

Disable Check Char Transmission

| Start / End |  |
| --- | --- |
| Check Character Algorithm |  |
| MOD 10 / DEFAULT | MOD 10 / MOD 10 |
| MOD 10 / MOD 11 |  |

Set Lengths for MSI

- One Discrete Length

Select this option to decode the symbol containing a selected length.

Select the length using the numeric bar codes in ASCII Code. For example, to decode only MSI symbols with 14 characters, scan MSI One Discrete Length, then scan 1 followed by 4.

- Two Discrete Lengths

Select this option to decode the symbol containing either of two selected lengths. Select lengths using the numeric bar codes in ASCII Code. For example, to decode only MSI symbols containing either 2 or 14 characters, select MSI Two Discrete Lengths, then scan 0, 2, 1, and then 4.

- Length Within Range

Select this option to decode the symbol with a specific length range.

Select lengths using numeric bar codes in ASCII Code. For example, to decode

MSI symbols containing between 4 and 12 characters, first scan MSI Length Within Range. Then scan 0, 4, 1, and 2.

- Any Length

Select this option to decode the symbol containing any number of characters within the digital scanner’s capability.

| Start / End |  |
| --- | --- |
| Set Lengths for MSI - Continued |  |
| One Discrete Length | Two Discrete Lengths |
| Length Within Range | Any Length / DEFAULT |

Interleaved 2 of 5

Start / End

Enable/Disable Interleaved 2 of 5

Enable Interleaved 2 of 5 / DEFAULT

Disable Interleaved 2 of 5

Start / End

Check Character Calculation

Disable Check Char Calculation / DEFAULT

Enable Check Char Calculation

Start / End

Check Character Transmission

Disable Check Char Transmission / DEFAULT

Enable Check Char Transmission

Set Lengths for Interleaved 2 of 5

- One Discrete Length

Select this option to decode the symbol containing a selected length.

Select the length using the numeric bar codes in ASCII Code. For example, to decode only Interleaved 2 of 5 symbols with 14 characters, scan Interleaved 2 of 5 One Discrete Length, then scan 1 followed by 4.

- Two Discrete Lengths

Select this option to decode the symbol containing either of two selected lengths. Select lengths using the numeric bar codes in ASCII Code. For example, to decode only Interleaved 2 of 5 symbols containing either 2 or 14 characters, select Interleaved 2 of 5 Two Discrete Lengths, then scan 0, 2, 1, and then 4.

- Length Within Range

Select this option to decode the symbol with a specific length range.

Select lengths using numeric bar codes in ASCII Code. For example, to decode

Interleaved 2 of 5 symbols containing between 4 and 12 characters, first scan Interleaved 2 of 5 Length Within Range. Then scan 0, 4, 1, and 2.

- Any Length

Select this option to decode the symbol containing any number of characters within the digital scanner’s capability.

| Start / End |  |
| --- | --- |
| Set Lengths for Interleaved 2 of 5 - Continued |  |
| One Discrete Length | Two Discrete Lengths |
| Length Within Range | Any Length / DEFAULT |

GS1 DataBar 14

Start / End

Enable/Disable GS1 DataBar 14

Enable GS1 DataBar 14 / DEFAULT

Disable DataBar 14

Start / End

Application Identifier Transmission

Enable Application Identifier “01” Transmission/ DEFAULT

Disable Application Identifier “01” Transmission

GS1 DataBar 14 Stacked

Start / End

Enable/Disable GS1 DataBar 14 Stacked

Enable GS1 DataBar 14 Stacked / DEFAULT

Disable GS1 DataBar 14 Stacked

Start / End

Application Identifier Transmission

Enable Application Identifier “01” Transmission/ DEFAULT

Disable Application Identifier “01” Transmission

GS1 DataBar Expanded

Start / End

Enable/Disable GS1 DataBar Expanded

Enab Enable GS1 DataBar Expanded / DEFAULT

Disable GS1 DataBar Expanded

Start / End

Application Identifier Transmission

Enable Application Identifier “01” Transmission/ DEFAULT

Disable Application Identifier “01” Transmission

GS1 DataBar Expanded Stacked

Start / End

Enable/Disable GS1 DataBar Expanded Stacked

Enable GS1 DataBar Expanded Stacked / DEFAULT

Disable GS1 DataBar Expanded Stacked

Start / End

Application Identifier Transmission

Enable Application Identifier “01” Transmission/ DEFAULT

Disable Application Identifier “01” Transmission

GS1 DataBar Limited

Start / End

Enable/Dsiable GS1 DataBar Limited

Enable GS1 DataBar Limited / DEFAULT

Disable GS1 DataBar Limited

Start / End

Application Identifier Transmission

Enable Application Identifier “01” Transmission/ DEFAULT

Disable Application Identifier “01” Transmission

GS1 Composite Component A

Start / End

Enable/Disable GS1 Composite Component A

Enable CC-A

Disable CC-A / DEFAULT

GS1 Composite Component B

Start / End

Enable/Disable GS1 Composite Component B

Enable CC-B

Disable CC-B / DEFAULT

GS1 Composite Component C

Start / End

Enable/Disable GS1 Composite Component C

Enable CC-C

Disable CC-C / DEFAULT

PDF417

Start / End

Enable/Disable PDF417

Enable PDF417 / DEFAULT

Disable PDF417

Micro PDF417

Start / End

Enable/Disable Micro PDF417

Enable Micro PDF417

Disable Micro PDF417 / DEFAULT

Data Matrix

Start / End

Enable/Disable Data Matrix

Enable Data Matrix / DEFAULT

Disable Data Matrix

Start / End

GS1 Data Matrix AIM ID

Transmit GS1 Data Matrix AIM ID

Do Not Transmit GS1 Data Matrix AIM ID / DEFAULT

QR

Start / End

Enable/Disable QR

Enable QR / DEFAULT

Disable QR

Start / End

GS1 QR AIM ID

Transmit GS1 QR AIM ID

Do Not Transmit GS1 QR AIM ID / DEFAULT

Start / End

Unicode Keyboard Output

Microsoft O Unicode - UTF8 Keyboard Output / DEFAULT

Microsoft Office Word - UTF8 / Codepage Keyboard Output

Start / End

Unicode Keyboard Output - Continued

GB2312 - QR Keyboard Output

GB2312 - Microsoft Office Word

Start / End

Unicode Keyboard Output - Continued

Big 5 - QR Keyboard Output

Big 5 - Microsoft Office Word

Start / End

Unicode Keyboard Output - Continued

Shift JIS - QR Keyboard Output

Shift JIS - Microsoft Office Word

Start / End

Unicode Keyboard Output - Continued

Thai - UTF8 QR Keyboard Output

Russian - UTF8 QR Keyboard Output

Start / End

Unicode Keyboard Output - Continued

Turkish - UTF8 QR Keyboard Output

German - UTF8 QR Keyboard Output

Start / End

Unicode Keyboard Output - Continued

Dutch - UTF8 QR Keyboard Output

Italian - UTF8 QR Keyboard Output

Start / End

Unicode Keyboard Output - Continued

French - UTF8 QR Keyboard Output

Spanish - UTF8 QR Keyboard Output

Start / End

Unicode Keyboard Output - Continued

Portuguese - UTF8 QR Keyboard Output

Brazil - UTF8 QR Keyboard Output

Start / End

Unicode Keyboard Output - Continued

Arabic - UTF8 QR Keyboard Output

Malaysia - UTF8 QR Keyboard Output

Micro QR

Start / End

Enable/Disable Micro QR

Enable Micro QR

Disable Micro QR / DEFAULT

Aztec

Start / End

Enable/Disable Aztec

Enable Aztec

Disable Aztec / DEFAULT

MaxiCode

Start / End

Enable/Disable MaxiCode

Enable MaxiCode

Disable MaxiCode / DEFAULT

DotCode

Start / End

Enable/Disable DotCode

Enable DotCode

Disable DotCode / DEFAULT

Start / End

GS1 DotCode AIM ID

Transmit GS1 DotCode AIM ID

Do Not Transmit GS1 DotCode AIM ID / DEFAULT

Data Editing

Data Format

The scan data is transmitted as below format.

| Prefix | AIM ID | Scan Data | Suffix |
| --- | --- | --- | --- |

Prefix/Suffix

One to six prefixes and/or suffixes can be appended to scan data for use in data editing.

Example:

Set two Prefixes/Suffixes for all codes

<Enter programming Mode>

<Set Prefix> or <Set Suffix>

<Set All Codes>

<Set first code of ASCII Codes or Direct Keys >

<Set second code of ASCII Codes or Direct Keys >

<Exit programming Mode>

Disable Prefixes/Suffixes for all codes

<Enter programming Mode>

<Disable Prefix> or <Disable Suffix>

<Set All Codes>

<Exit programming Mode>

Start / End

Set Prefix - Continued

Set Prefix

Disable Prefix / DEFAULT

Start / End

Set Suffix - Continued

Set Suffix (Default CR for all codes)

Disable Suffix

Truncate Data

This parameter sets the number of leading or ending data to be truncated. It is programmable in 1 increment from 1 to 99 characters. The default setting is 0.

Truncate 5 characters of leading for all codes

1. Scan the Start barcode.

2. Scan the Truncate Leading barcode.

2. Scan the All Codes barcode.

3. Scan the “5” barcode from the Digit Number in Appendix C.

4. Scan the End barcode.

Truncate 8 characters of Leading for Data Matrix

1. Scan the Start barcode.

2. Scan the Truncate Leading barcode.

2. Scan the Data Matrix barcode.

3. Scan the “8” barcode from the Digit Number in Appendix C.

4. Scan the End barcode.

Truncate 10 characters of Ending for QR

1. Scan the Start barcode.

2. Scan the Truncate Leading barcode.

2. Scan the QR barcode.

3. Scan the “1” and “0” barcodes from the Digit Number in Appendix C.

4. Scan the End barcode.

Start / End

Truncate Data - Continued

Truncate Leading

Truncate Ending

| Start / End |  |
| --- | --- |
| Set Data for Codes |  |
| Set All Codes | UPC-A |
| UPC-E | EAN 13 |

| Start / End |  |
| --- | --- |
| Set Data for Codes - Continued |  |
| EAN 8 | Code 128 |
| Code 39 | Code 93 |

| Start / End |  |
| --- | --- |
| Set Data for Codes - Continued |  |
| Code 32 | Pharmacode |
| Codabar | MSI |

| Start / End |  |
| --- | --- |
| Set Data for Codes - Continued |  |
| Interleaved 2 of 5 | GS1 DataBar 14 |
| GS1 DataBar 14 Stacked | GS1 DataBar Expanded |

| Start / End |  |
| --- | --- |
| Set Data for Codes - Continued |  |
| GS1 DataBar Expanded Stacked | GS1 DataBar Limited |
| CC-A | CC-B |

| Start / End |  |
| --- | --- |
| Set Data for Codes - Continued |  |
| CC-C | PDF417 |
| Micro PDF417 | Data Matrix |

| Start / End |  |
| --- | --- |
| Set Data for Codes - Continued |  |
| QR | Micro QR |
| Aztec | MaxiCode |

Start / End

AIM ID

Disable Transmission of AIM ID / DEFAULT

Enable Transmission of AIM ID

| Start / End |  |
| --- | --- |
| Keyboard Function Key Mapping Enable this to send the keys in bold (see ASCII Codes) in place of the standard key mapping. Table entries that do not have a bold entry remain the same whether or not this parameter is enabled |  |
| Disable Function Key Mapping |  |
| Enable Function Key Mapping / DEFAULT |  |
| Start / End |  |
| Keyboard Function Key Mapping - Continued |  |
| Ctrl + ASCII Mode / DEFAULT Control Characters (0x00 - 0x1F) are sent as ASCII sequences. |  |
| Alt + Numeric Keypad Mode Control Characters (0x00 - 0x1F) are sent as Unicode code sequences. |  |

Start / End

Keyboard Caps Lock State

Caps Lock Off / DEFAULT

Caps Lock On

| Start / End |  |
| --- | --- |
| Case Conversion |  |
| Disable / DEFAULT | Convert to Upper Case |
| Convert to Lower Case |  |

Note: Case Conversion does not affect AIM ID , Prefix , Suffix.

Control Characters Conversion

Convert Control Characters (0x00 - 0x1F) to other keystroke.

Set ASCII value 29 [GS] to #

1. Scan the Start barcode.

2. Scan the GS Conversion barcode.

3. Scan the # barcode from the ASCII Code in Appendix A.

4. Scan the End barcode.

Disable ASCII value 29 [GS] conversion

1. Scan the Start barcode.

2. Scan the Disable GS Conversion barcode.

4. Scan the End barcode.

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| NUL Conversion | Disable NUL Conversion |
| SOH Conversion | Disable SOH Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| STX Conversion | Disable STX Conversion |
| ETX Conversion | Disable ETX Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| EOT Conversion | Disable EOT Conversion |
| ENQ Conversion | Disable ENQ Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| ACK Conversion | Disable ACK Conversion |
| BEL Conversion | Disable BEL Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| BS Conversion | Disable BS Conversion |
| HT Conversion | Disable HT Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| LF Conversion | Disable LF Conversion |
| VT Conversion | Disable VT Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| FF Conversion | Disable FF Conversion |
| CR Conversion | Disable CR Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| SO Conversion | Disable SO Conversion |
| SI Conversion | Disable SI Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| DLE Conversion | Disable DLE Conversion |
| DC1 Conversion | Disable DC1 Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| DC2 Conversion | Disable DC2 Conversion |
| DC3 Conversion | Disable DC3 Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| DC4 Conversion | Disable DC4 Conversion |
| NAK Conversion | Disable NAK Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| SYN Conversion | Disable SYN Conversion |
| ETB Conversion | Disable ETB Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| CAN Conversion | Disable CAN Conversion |
| EM Conversion | Disable EM Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| SUB Conversion | Disable SUB Conversion |
| ESC Conversion | Disable ESC Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| FS Conversion | Disable FS Conversion |
| GS Conversion | Disable GS Conversion |

| Start / End |  |
| --- | --- |
| Control Characters Conversion - Continued |  |
| RS Conversion | Disable RS Conversion |
| US Conversion | Disable US Conversion |

| Appendix A - ASCII Codes |  |  |  |
| --- | --- | --- | --- |
| ASCII (hex) | Serial | Keystroke |  |
| 01 | SOH | ENTER / CTRL+A |  |
| 02 | STX | F11 / CTRL+B |  |
| 03 | ETX | F12 / CTRL+C |  |
| 04 | EOT | NULL / CTRL+D |  |
| The keystroke in bold is sent only if Function Key Mapping is enabled. Otherwise, the unbolded keystroke is sent. |  |  |  |

| ASCII Codes - Continued |  |  |  |
| --- | --- | --- | --- |
| ASCII (hex) | Serial | Keystroke |  |
| 05 | ENQ | NULL / CTRL+E |  |
| 06 | ACK | NULL / CTRL+F |  |
| 07 | BEL | NULL / CTRL+G |  |
| 08 | BACKSPACE | BACKSPACE |  |
| The keystroke in bold is sent only if Function Key Mapping is enabled. Otherwise, the unbolded keystroke is sent. |  |  |  |

| ASCII Codes - Continued |  |  |  |
| --- | --- | --- | --- |
| ASCII (hex) | Serial | Keystroke |  |
| 09 | TAB | TAB |  |
| 0A | LF | RIGHT / CTRL+J |  |
| 0B | VT | NULL / CTRL+K |  |
| 0C | FF | NULL / CTRL+L |  |
| The keystroke in bold is sent only if Function Key Mapping is enabled. Otherwise, the unbolded keystroke is sent. |  |  |  |

| ASCII Codes - Continued |  |  |  |
| --- | --- | --- | --- |
| ASCII (hex) | Serial | Keystroke |  |
| 0D | CR | ENTER |  |
| 0E | SO | INSERT / CTRL+N |  |
| 0F | SI | Page Up / CTRL+O |  |
| 10 | DLE | Page Down / CTRL+P |  |
| The keystroke in bold is sent only if Function Key Mapping is enabled. Otherwise, the unbolded keystroke is sent. |  |  |  |

| ASCII Codes - Continued |  |  |  |
| --- | --- | --- | --- |
| ASCII (hex) | Serial | Keystroke |  |
| 11 | DC1 | HOME / CTRL+Q |  |
| 12 | DC2 | LEFT / CTRL+R |  |
| 13 | DC3 | DWON / CTRL+S |  |
| 14 | DC4 | Up / CTRL+T |  |
| The keystroke in bold is sent only if Function Key Mapping is enabled. Otherwise, the unbolded keystroke is sent. |  |  |  |

| ASCII Codes - Continued |  |  |  |
| --- | --- | --- | --- |
| ASCII (hex) | Serial | Keystroke |  |
| 15 | NAK | F6 / CTRL+U |  |
| 16 | SYN | F1 / CTRL+V |  |
| 17 | ETB | F2 / CTRL+W |  |
| 18 | CAN | F3 / CTRL+X |  |
| The keystroke in bold is sent only if Function Key Mapping is enabled. Otherwise, the unbolded keystroke is sent. |  |  |  |

| ASCII Codes - Continued |  |  |  |  |
| --- | --- | --- | --- | --- |
| ASCII (hex) | Serial | Keystroke |  |  |
| 19 | EM | F4 / CTRL+Y |  |  |
| 1A | SUB | F5 / CTRL+Z |  |  |
| 1B | ESC | ESC / CTRL+[ |  |  |
| 1C | FS | F7 / CTRL+\ |  |  |
| The keystroke in bold is sent only if Function Key Mapping is enabled. Otherwise, the unbolded keystroke is sent. |  |  |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 1D | GS | F8 / CTRL+] |  |  |
| 1E | RS | F9 / CTRL+^ |  |  |
| 1F | US | F10 / CTRL+_ |  |  |
| 20 | SPACE | SPACE |  |  |
| The keystroke in bold is sent only if Function Key Mapping is enabled. Otherwise, the unbolded keystroke is sent. |  |  |  |  |

| ASCII Codes - Continued |  |  |  |  |
| --- | --- | --- | --- | --- |
| ASCII (hex) | Serial | Keystroke |  |  |
| 21 | ! | ! |  |  |
| 22 | " | " |  |  |
| 23 | # | # |  |  |
| 24 | $ | $ |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 25 | % | % |  |  |
| 26 | & | & |  |  |
| 27 | ' | ' |  |  |
| 28 | ( | ( |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 29 | ) | ) |  |  |
| 2A | * | * |  |  |
| 2B | + | + |  |  |
| 2C | , | , |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 2D | - | - |  |  |
| 2E | . | . |  |  |
| 2F | / | / |  |  |
| 30 | 0 | 0 |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 31 | 1 | 1 |  |  |
| 32 | 2 | 2 |  |  |
| 33 | 3 | 3 |  |  |
| 34 | 4 | 4 |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 35 | 5 | 5 |  |  |
| 36 | 6 | 6 |  |  |
| 37 | 7 | 7 |  |  |
| 38 | 8 | 8 |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 39 | 9 | 9 |  |  |
| 3A | : | : |  |  |
| 3B | ; | ; |  |  |
| 3C | < | < |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 3D | = | = |  |  |
| 3E | > | > |  |  |
| 3F | ? | ? |  |  |
| 40 | @ | @ |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 41 | A | A |  |  |
| 42 | B | B |  |  |
| 43 | C | C |  |  |
| 44 | D | D |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 45 | E | E |  |  |
| 46 | F | F |  |  |
| 47 | G | G |  |  |
| 48 | H | H |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 49 | I | I |  |  |
| 4A | J | J |  |  |
| 4B | K | K |  |  |
| 4C | L | L |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 4D | M | M |  |  |
| 4E | N | N |  |  |
| 4F | O | O |  |  |
| 50 | P | P |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 51 | Q | Q |  |  |
| 52 | R | R |  |  |
| 53 | S | S |  |  |
| 54 | T | T |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 55 | U | U |  |  |
| 56 | V | V |  |  |
| 57 | W | W |  |  |
| 58 | X | X |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 59 | Y | Y |  |  |
| 5A | Z | Z |  |  |
| 5B | [ | [ |  |  |
| 5C | \ | \ |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 5D | ] | ] |  |  |
| 5E | ^ | ^ |  |  |
| 5F | _ | _ |  |  |
| 60 | ' | ' |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 61 | a | a |  |  |
| 62 | b | b |  |  |
| 63 | c | c |  |  |
| 64 | d | d |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 65 | e | e |  |  |
| 66 | f | f |  |  |
| 67 | g | g |  |  |
| 68 | h | h |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 69 | i | i |  |  |
| 6A | j | j |  |  |
| 6B | k | k |  |  |
| 6C | l | l |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 6D | m | m |  |  |
| 6E | n | n |  |  |
| 6F | o | o |  |  |
| 70 | p | p |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 71 | q | q |  |  |
| 72 | r | r |  |  |
| 73 | s | s |  |  |
| 74 | t | t |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 75 | u | u |  |  |
| 76 | v | v |  |  |
| 77 | w | w |  |  |
| 78 | x | x |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 79 | y | y |  |  |
| 7A | z | z |  |  |
| 7B | { | { |  |  |
| 7C | ¦ | ¦ |  |  |
| ASCII Codes - Continued |  |  |  |  |
| ASCII (hex) | Serial | Keystroke |  |  |
| 7D | } | } |  |  |
| 7E | ~ | ~ |  |  |

Appendix B - Direct Keys

F1

F2

F3

F4

Direct Keys - Continued

F5

F6

F7

F8

Direct Keys - Continued

F9

F10

F11

F12

Direct Keys - Continued

INSERT

DELETE

HOME

END

Direct Keys - Continued

Arrow Up

Arrow Down

Arrow Left

Arrow Right

Direct Keys - Continued

CTRL

ALT

SHIFT

Page Up

Direct Keys - Continued

Page Down

ALT+a

ALT+b

ALT+c

Direct Keys - Continued

ALT+d

ALT+e

ALT+f

ALT+g

Direct Keys - Continued

ALT+h

ALT+i

ALT+j

Ctrl+k

Direct Keys - Continued

ALT+l

ALT+m

ALT+n

ALT+o

Direct Keys - Continued

ALT+p

ALT+q

ALT+r

ALT+s

Direct Keys - Continued

ALT+t

ALT+u

ALT+v

ALT+w

Direct Keys - Continued

ALT+x

ALT+y

ALT+z

| Appendix C - Digit Number |  |
| --- | --- |
| 0 | 1 |
| 2 | 3 |
| 4 | 5 |
| 6 | 7 |
| 8 | 9 |